HW3 Calibrated Controlled Propeller

Johns Hopkins University

Real Time Software for Embedded Systems

Fall 2014

Tony Florida

2014-10-07

Requirements

Hardware

- There shall be at least one IR led emitter and detector circuit

- There shall be two motor that are spun via electronic speed control (ESC)

- An Arduino board shall control the circuits

- External battery supply shall be used for the motors

- Blades shall be attached to at least one motor and pass through an emitter/detector pair

Software

- The software running on the Arduino shall use function queue scheduling design

- The software shall capture the rotations per minute (RPM) every second

- The software shall capture the command every second

- The software shall capture the time every second

- The software shall command the motors via the ESC from min rotation speed to max, then back down to min

Parts List

- (1) Arduino Uno

- (1) 10k resistor

- (1) 220 ohm resistor

- (1) spool of hobby wire

- (1) USB 2.0 A/B cable

- (1) breadboard

- (1) 12v battery preferable Zippy Compact 25c Series 4000 Li-Po Battery

- (1) XT60 Connector Pair

- (1) LED of any color

- (1) Infrared Emitter and Detector (Radio shack 276-142)

- (2) Turnigy Multistar 30 Amp Multi-rotor Brushless ESC 2-4S

- (1) Gemfan 9x4.7 Nylon Prop Set (1x CW & 1x CCW)

- (2) Turnigy Multistar ESC Programming Card

- Wood to hold motor and emitter/detector

- Nails

- Twist ties

- Black electrical tape

Required Software

- Arduino Sketch v1.0

- DuinOS v0.4

- AVRQueue

- Microsoft Excel 2010

Architecture

Hardware

Figure 1 - Circuit Schematic [1]

Figure 2 – Schematic Notes [1]

Figure 3 – Emitter Detector Sensors

Software

Figure 4 - Software Architecture Diagram

Figure 5 - Hardware/Software Sequence Diagram

Design

Software

Photos of the Hardware

Figure 6 – Tactical Test Setup

Figure 7 - Top View of the Test Setup

Implementation

#include <Queue.h>

#include <Servo.h>

//JHU RTSW HW 3 - HW3 - Calibrated Controlled Propeller

//Tony Florida

//2014-10-06

//References:

// http://www.instructables.com/id/Arduino-Based-Optical-Tachometer/

// http://techvalleyprojects.blogspot.com/2012/06/arduino-control-escmotor-tutorial.html

// https://github.com/Zuph/AVRQueue

//CMD variables (range of 15 to 150)

int serial_cmd = 5; //serial commands start at 10

int MAX_SERIAL_CMD = 100; //max serial command

int SERIAL_CMD_INCREMENT = 5; //increment serial commands by 20

//Queue variables

Queue myQueue;

//ESC variables

// This is our motor.

Servo myMotor;

Servo myMotor2;

// This is the final output

// written to the motor.

String incomingString;

//IR Emitter Detector variables

int ledPin = 13;                // IR LED connected to digital pin 13

int statusPin = 12;             // LED connected to digital pin 12

volatile byte rpmcount;

volatile int status;

unsigned int rpm;

unsigned long timeold;

//IR Emitter Detector function

void rpm_fun()

{

//Each rotation, this interrupt function is run twice, so take that into consideration for

//calculating RPM

//Update count

rpmcount++;

//Toggle status LED

if (status == LOW) {

status = HIGH;

} else {

status = LOW;

}

digitalWrite(statusPin, status);

}

int cmd()

{

serial_cmd+=SERIAL_CMD_INCREMENT; //increment the serial command

if(serial_cmd > MAX_SERIAL_CMD)

{

SERIAL_CMD_INCREMENT *= -1; //incrementally spin down the motors

}

if(serial_cmd <= 0)

{

// we are done!

myQueue.scheduleRemoveFunction("ESC");

}

return serial_cmd;

}

void setup() {

// Required for I/O from Serial monitor

Serial.begin(9600);

//ESC setup

Serial.println("Initializing ESC");

// Put the motors to Arduino pin 9 and 10

myMotor.attach(9);

myMotor2.attach(10);

//IR Emitter Detector setup

//Interrupt 0 is digital pin 2, so that is where the IR detector is connected

//Triggers on FALLING (change from HIGH to LOW)

attachInterrupt(0, rpm_fun, FALLING);

//Turn on IR LED

pinMode(ledPin, OUTPUT);

digitalWrite(ledPin, HIGH);

//Use statusPin to flash along with interrupts

pinMode(statusPin, OUTPUT);

rpmcount = 0;

rpm = 0;

timeold = 0;

status = LOW;

//Function queue scheduling setup

Serial.println("Initializing function queue scheduling");

myQueue.scheduleFunction(esc, "ESC", 0, 3000);

//Print table header

Serial.println("Time(ms),RPM,Command");

//Wait until start command i.e. any input serial comms

Serial.println("Plug battery into motors, then send any serial command");

while(!Serial.available()) {}

while(1) {

myQueue.Run(millis());

rpm_counter();

}

}

//Receive ESC commands via serial

int esc(unsigned long now)

{

int val = cmd(); //new rotation speed

/*  We only want to write an integer between

*  0 and 180 to the motor.

*/

if (val > -1 && val < 181)

{

// Print confirmation that the

// value is between 0 and 180

// Write to Servo

myMotor.write(val);

myMotor2.write(val);

}

}

//Count RPMs

void rpm_counter()

{

//Update RPM every second

delay(1000);

//Don't process interrupts during calculations

detachInterrupt(0);

//Note that this would be 60*1000/(millis() - timeold)*rpmcount if the interrupt

//happened once per revolution instead of twice. Other multiples could be used

//for multi-bladed propellers or fans

rpm = 30*1000/(millis() - timeold)*rpmcount;

timeold = millis();

rpmcount = 0;

//Write it out to serial port

Serial.print(millis());

Serial.print(",");

Serial.print(rpm,DEC);

Serial.print(",");

Serial.println(serial_cmd);

//Restart the interrupt processing

attachInterrupt(0, rpm_fun, FALLING);

}

//not using the loop in this program

void loop() {

}

Results

Blade 1

Time(ms)	RPM	Command

22408	0	25

23409	0	25

24410	0	25

25410	900	30

26411	638	30

27412	435	30

28412	1020	35

29414	1334	35

30414	1440	35

31415	2001	40

32416	2262	40

33417	2233	40

34418	2755	45

35419	2900	45

36419	3000	45

37421	3364	50

38421	3570	50

39422	3451	50

40423	3799	55

41424	3857	55

42425	3857	55

43425	4290	60

44427	4205	60

45427	4350	60

46429	4495	65

47429	4740	65

48430	4553	65

49431	4901	70

50432	5220	70

51433	4959	70

52433	5400	75

53435	5278	75

54435	5520	75

55436	5423	80

56437	5539	80

57438	5539	80

58439	5858	85

59440	5800	85

60440	5970	85

61442	6061	90

62442	6570	90

63443	1218	90

64444	6467	95

65445	6690	95

66446	6554	95

67446	690	100

68448	87	100

69448	7620	100

70450	7366	105

71450	7500	105

72451	7192	105

73452	7221	100

74452	7080	100

75454	232	100

76454	6720	95

77456	6438	95

78456	6660	95

79458	6119	90

80458	6270	90

81459	6061	90

82460	5800	85

83461	5771	85

84462	5742	85

85463	5510	80

86463	5640	80

87464	5481	80

88465	5394	75

89466	5249	75

90467	5278	75

91468	4988	70

92469	4959	70

93469	5100	70

94471	4640	65

95471	4740	65

96473	4582	65

97473	4410	60

98473	4380	60

99475	4234	60

100475	4050	55

101477	3828	55

102477	3990	55

103479	3538	50

104479	3570	50

105481	3451	50

106481	3150	45

107482	2900	45

108483	2871	45

109484	2465	40

110485	2291	40

111485	2370	40

112487	1711	35

113487	1470	35

114489	1363	35

115489	960	30

116491	522	30

117491	420	30

118492	116	25

119493	0	25

120494	0	25

Blade 2

Time(ms)	RPM	Command

17783	0	25

18784	0	25

19784	0	25

20786	928	30

21786	660	30

22787	464	30

23788	986	35

24788	1350	35

25789	1363	35

26790	2030	40

27791	2233	40

28792	2340	40

29793	2726	45

30793	3000	45

31795	2900	45

32795	3450	50

33797	3451	50

34797	3570	50

35799	3799	55

36799	3990	55

37799	4020	55

38801	4147	60

39801	4410	60

40803	4234	60

41803	4680	65

42804	4582	65

43805	4582	65

44806	4901	70

45807	4988	70

46808	4959	70

47808	5430	75

48809	5249	75

49810	5278	75

50811	5452	80

51812	5452	80

52812	5670	80

53814	5771	85

54814	6000	85

55816	5800	85

56816	6240	90

57818	6090	90

58818	6270	90

59819	6409	95

60820	6438	95

61820	6660	95

62822	6757	100

63822	7050	100

64824	6786	100

65824	7320	105

66826	7105	105

67826	7320	105

68828	6815	100

69828	7020	100

70829	6757	100

71830	6438	95

72830	6630	95

73832	6409	95

74832	6270	90

75834	6032	90

76834	6240	90

77835	5742	85

78836	5742	85

79837	5713	85

80838	5452	80

81839	5452	80

82839	5610	80

83841	5249	75

84841	5400	75

85842	5249	75

86843	4959	70

87843	5100	70

88845	4930	70

89845	4770	65

90847	4553	65

91847	4710	65

92849	4263	60

93849	4350	60

94850	4234	60

95851	3915	55

96851	3990	55

97853	3857	55

98853	3660	50

99854	3451	50

100855	3451	50

101856	3016	45

102857	2900	45

103858	2900	45

104859	2436	40

105860	2291	40

106861	2291	40

107862	1711	35

108863	1392	35

109863	1440	35

110865	928	30

111865	540	30

112866	435	30

113867	87	25

114868	0	25

115868	0	25

Plot

Video Presentation

https://www.youtube.com/watch?v=vAZy4O3XNxI

References

[1] http://www.instructables.com/id/Arduino-Based-Optical-Tachometer/

[2] http://techvalleyprojects.blogspot.com/2012/06/arduino-control-escmotor-tutorial.html

[3] https://github.com/Zuph/AVRQueue

[4] https://github.com/DuinOS/DuinOS